# Networking Basics - Screenshot Notes

# Block 1

Mumshad Mannambeth continues with the session. And now let us look at some networking basics, such as switching and routing.

# Block 2

In this section, we will focus on general networking concepts, such as network interfaces, gateways, and routes. We have two computers, A and B, laptops,

# Block 3

desktops, VMs on the cloud, wherever. How does system A reach B? We connect them to a switch, and the switch creates

# Block 4

a network connecting the two systems. To connect them to a switch, we need an interface on each host, physical or virtual, depending on the host.

# Block 5

To see the interfaces for the host, we use the ip link command. In this case, we look at the interface named eth0 that we

# Block 6

will be using to connect to the switch. Let's assume it's a network with the address one ninety-two dot one sixty-eight dot one dot zero.

# Block 7

We then assign the systems IP addresses on the same network. For this, we use the command ip addr. Once the links are up and the IP addresses are assigned, the

# Block 8

computers can now communicate with each other through the switch. The switch can only enable communication within a network. This means it can receive packets from a host on the network

# Block 9

and deliver them to other systems within the same network. Say we have another network containing systems C and D at address 192.168.2.0.

# Block 10

The systems have IP addresses 192.168.2.10 and 192.168.2.11 respectively. How does a system in one network reach a system in the other?

# Block 11

How does system B with the IP one dot one one

reach system C with the IP two dot one zero on the

other network?

# Block 12

That's where a router comes in. A router helps

connect two networks together. It is an intelligent

device, so think of it as

# Block 13

another server with many network ports. Since it

connects to the two separate networks, it gets two

IPs assigned, one on each network.

# Block 14

In the first network, we assign it an IP address one

point one, and in the second one, we assign it

# Block 15

an IP address two point one. Now we have a router

connected to the two networks that can enable

communication between them.

# Block 16

Now, when system B tries to send a packet to

system C, how does it know where the router is on

the

# Block 17

network to send the packet through? The router is

just another device on the network. There could be

many other such devices.

# Block 18

That's where we configure the system with a

gateway or a route. If the network was a room, the

gateway is a door to

# Block 19

the outside world, to other networks, or to the

Internet. The systems need to know where the door

is to go through that door.

# Block 20

To see the existing routing configuration on a

system, run the route command. It displays the

kernel's routing table. And within that, as you can

see, there are

# Block 21

no routing configurations as of now. So in this

condition, your system B will not be able to reach

system C.

# Block 22

It can only reach other systems within the same

network in the range one ninety-two dot one sixty-

eight dot one dot zero.

# Block 23

In Linux, to configure a gateway on system B to

reach the systems on network two point zero, run

the ip route add command

# Block 24

and specify that you can reach the two point zero

network through the router or gateway at one point

one.

# Block 25

Rerunning the route command shows that we have

a route added to reach the two dot zero network

through the router.

# Block 26

Now remember, this has to be configured on all

systems. For example, if system C is to send a

packet to system

# Block 27

B, then you need to add a route on system C's

routing table to access network one point zero

through the router configured with

# Block 28

the IP address two point one. Now suppose these

systems need access to the Internet. Say they need

access to Google at one seventy-two dot two

seventeen

# Block 29

dot one ninety-four dot zero network on the Internet,

so you connect the router to the Internet. And then,

add a new route in your routing table to route

# Block 30

all traffic to the network one seventy-two point two

seventeen point one ninety-four through your router.

There are so many different sites on different

networks on the internet.

# Block 31

Instead of adding a routing table entry with the

same router's IP address for each of those

networks, you can simply say, for any

# Block 32

network that you don't know a route to, use this

router as the default gateway. This way, any request

to any network outside of your existing

# Block 33

network goes to this particular router. So in a simple

setup like this, all you need is a single routing table

entry with the default gateway

# Block 34

set to the router's IP address. Remember, instead of

the word default, you could also say zero dot zero

dot zero dot zero.

# Block 35

It means any IP destination. Both of these lines

mean the same thing. A zero zero zero zero entry in

the gateway field indicates

# Block 36

that you don't need a gateway. For example, in this

case, for system C to access any device in the two

point zero network, it doesn't need a gateway,

# Block 37

because it is its own network. But say you have

multiple routers in your network, one for the internet

and another for internal private networks.

# Block 38

Then you will need to have two separate entries,

one for each network. one entry for the internal

private network, and another default gateway for

# Block 39

all other networks, including public networks. So if

you are having issues reaching the internet from

your systems, this routing table and the default

gateway configuration are

# Block 40

a good place to look at. So let's take away some

key commands from this session. These will be

handy in our troubleshooting session.

# Block 41

ip link is to list and modify interfaces on the host. ip

addr command is to see the IP addresses assigned

to those interfaces.

# Block 42

ip addr add command is used to set IP addresses

on the interfaces. Now remember, changes made

using these commands are

# Block 43

only valid till the system restarts. If you want to

persist these changes, you must set them in the

slash et cetera slash network slash interfaces file.

# Block 44

ip route, or simply the route command, is used to

view the routing table, and ip route add command is

used to

# Block 45

add entries into the routing table.